| Cruise name | HPLC equipment/system (incl. column type) | Extraction method | Other details | References |
| --- | --- | --- | --- | --- |
| Cruises on RV Oceania in summer seasons between 1999 and 2024 (AREX expeditions), RV Akademik Ioffe (spring 2002 and summer 2003), RV Polarstern (spring 2003), RV DANA (summer 2021), and RV Maria S. Merian (summer 2022). | The chromatographic system HP1200 (Agilent, Perlan Technologies) was equipped with diode array absorbance detector, fluorescence detector and C18 LichroCART™LiChrospher™ 100 RP18e (Merck) analytical column (dimension 250 x 4 mm, particle size 5 μm and pore size 100 Å). | Pigments were extracted from cells by mechanical grinding and sonication (2 min, 20 kHz, Cole Parmer, 4710 Series) in darkness conditions at 4 ºC for 2 hours using 90% acetone solution (Parsons et al. 1984). After centrifugation (20 min., 4 ºC, 3210xg, Beckman, GS-6R) to remove the filters and cellular debris, extract was subjected to the chromatographic analysis. | Pigments separation, qualification and quantification was performed according to methodology described in Stoń-Egiert and Kosakowska (2005) using a gradient mixture of methanol, 1 M ammonium acetate and acetone used as a mobile phase. Pigments detection was based on their retention time and absorbance spectra, while quantification base on an external standardization equation combining the chromatographic parameters of individual pigments with their concentration. Calibration of chromatographic system was performed using commercially available chlorophylls and carotenoids standards (The International Agency for 14C Determination DHI Institute for Water and Environment in Denmark). The documented measurement precision was 2.9% ± 1.5% and a recurrence error was 9.7%±6.4% (Stoń-Egiert et al., 2010). | (Parsons et al., 1984) (Stoń-Egiert and Kosakowska, 2005) (Ston-Egiert et al., 2010) |
| LIU_2015_PS93.2 | Waters 600 controller, 1525 pump OPTIMAS autosampler Waters PDA 2998 Fluorescense detector Waters 2475 Software: EMPOWER3 | Water samples were filtered on board through Whatman GF/F filters with a pore size of 0.7 µm and the filters were thermally shocked in liquid nitrogen and stored in the -80°C freezer. Then for the extraction we used a refrigerated centrifuge Sigma 3-16KL with Eppie-Rot (4°C, 1500xg, 5 min) and warm up <30min. Thermo Scientific Evolution 201 UV-Visible Spectrophotometer. Fill 100% aceton in reference cuvette of photometer, balance it with Auto Zero. Add Canthaxanthin (~0.0161 mg L-1) into sample cuvette (instead of acetone) - internal standard for correcting solvent and sample loss!). Measure absorption of Canthaxanthin-standard at 750nm and 474 nm. a(474) of standard should be ~0.5mg/L (value is later used for calculations of HPLC pigments). Make sure that the sample filters in the CryoVials have thawed. Take filters from the sample tubes into labeled 2ml-Apexvial and add 100% acetone to sample filters (with water in filter finally ~90% acetone). | The soluble organic phytoplankton pigment concentrations were determined using HPLC according to the method of Barlow et al. (1997) adjusted to our temperature-controlled instruments (Taylor et al. 2011). The pigment data were quality controlled according to Aiken et al. (2009). Uncertainties of our HPLC measurements were assessed from triplicate samples taken at several prior cruises and campaigns (data sets related to Taylor et al. 2011, Bracher et al. 2015, Zindler et al. 2013, Tilstone et al. 2020). For the different cruises the average deviation for HPLC TChl-a and the seven other pigments used in the diagnostic pigment analysis (DPA, see below) ranged from 5 to 8% with a standard deviation for triplicates between 1 to 11%. | Follows (Barlow et al., 1997), adapted for ocean and polar waters by (Hoffmann et al., 2006), adapted for our instruments by (Taylor et al., 2011), quality control following (Aiken et al., 2009). Citation to uncertainties: (Bracher et al., 2015), (Zindler et al., 2013), (Tilstone et al., 2020) |
| LIU_2016_PS99.1 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 |
| BRACHER_2017_PS107 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 |
| LIU_2016_PS99.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 | as LIU_2015_PS93.2 |
| BRACHER_2019_PS121 | Waters e2695:pump, column, autosampler, degasser Waters PDA 2998 (ex. 190-800 nm, 1.2 nm resol.) HPLC-column: Microsorb-MV C8 Software: EMPOWER 3 | Use refrigerated centrifuge Sigma 3-16KL with Eppie-Rot (4°C, 1500xg, 5 min) and warm up <30min. Thermo Scientific Evolution 201 UV-Visible Spectrophotometer. Fill 100% aceton in reference cuvette of photometer, balance it with Auto Zero. Add Canthaxanthin (~0.0161 mg L-1) into sample cuvette (instead of acetone) - internal standard for correcting solvent and sample loss!). Measure absorption of Canthaxanthin-standard at 750nm and 474 nm. a(474) of standard should be ~0.5mg/L (value is later used for calculations of HPLC pigments). Make sure that the sample filters in the CryoVials have thawed. Take filters from the sample tubes into labeled 2ml-Apexvial and add 100% acetone to sample filters (with water in filter finally ~90% acetone). | The soluble organic phytoplankton pigment concentrations were determined using HPLC according to the method of Barlow et al. (1997) adjusted to our temperature-controlled instruments (Taylor et al. 2011). The pigment data were quality controlled according to Aiken et al. (2009). Uncertainties of our HPLC measurements were assessed from triplicate samples taken at several prior cruises and campaigns (data sets related to Taylor et al. 2011, Bracher et al. 2015, Zindler et al. 2013, Tilstone et al. 2020). For the different cruises the average deviation for HPLC TChl-a and the seven other pigments used in the diagnostic pigment analysis (DPA, see below) ranged from 5 to 8% with a standard deviation for triplicates between 1 to 11%. | Follows (Barlow et al., 1997), adapted for open ocean and polar waters by (Hoffmann et al., 2006), adapted for our instruments by (Álvarez et al., 2022) Suppl. Material S1. Citation to uncertainties: (Bracher et al., 2015), (Zindler et al., 2013), (Tilstone et al., 2020) |
| BRACHER_2020_MSM93 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 |
| BRACHER_2021_PS126 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 |
| BRACHER_2022_PS131_ATWAICE | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 |
| BRACHER_2023_PS136_HAUSGARTEN | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 |
| BRACHER_2024_PS143.2 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 | as BRACHER_2019_PS121 |
| Cruises on the R/V Araon during summer seasons (2015–2020): ARA06B, ARA07B, ARA08B, ARA09B, ARA10B, ARA10B_Ice, and ARA11B. | Pigments were assessed using an Agilent 1200 series chromatographic system (Germany) equipped with a diode array detector and a fluorescence detector (Zapata et al., 2000). Detailed procedures are described in Lee et al. (2019, 2025). A 250 mm × 4.6 mm, 5 µm, C8 column (XDB-C8; Agilent, USA) was used for the pigment separation. | Water samples were collected from surface and subsurface chlorophyll maximum (SCM) layers using a 10 L PVC Niskin water sampler mounted on a CTD rosette system. For pigment analysis, 1–4-L seawater samples were filtered through a 47-mm GF/F filter (Whatman) and stored at −80 °C. Within 2–3 months of collection, pigments were extracted by adding 3 ml of 100% acetone, ultrasonicated for 30 seconds, and maintained at 4 °C in the dark for 15 hours. Debris was removed by filtration through a 0.45-μm Teflon syringe filter. Prior to injection, the extracts were diluted with distilled water (1 mL of extract + 0.3 mL of water) to avoid peak distortion of the initial eluting pigments. | Before the analysis, the instrument (series 1200 chromatographic system; Agilent, Germany) was calibrated using the standard pigments (DHI, Denmark). Pigments were identified by their retention times and absorbance spectra. The retention times were compared with those of pure standards and those reported in Zapata et al. (2000). | (Lee et al., 2019) (Lee et al., 2025) (Zapata et al., 2000) |
| Thielecke_PS138_Arc Watch | Pigments were quantified with a Waters Alliance 2695 HPLC Separation Module connected to a Waters photodiode array detector (2996 Waters Corporation, Milford, USA) using a Microsorb‐MV3 C8 column (4.6 × 100 mm), and an HPLC‐grade solvent using a modified method of Barlow et al. (1997). Details are described in Möller et al., 2024. | For analytical preparation, an internal standard (50 μL, canthaxanthin), 100% acetone HPLC grade (1.5 mL) and glass beads were added to each filter sample (on ice) and the samples were homogenized with small glass beads in a Precellys® tissue homogenizer and thereafter centrifuged (0°C). The supernatant liquid is filtered through a 0.2 μm PTFE filter (Rotilabo) and stored at -80°C prior to analysis. An aliquot of 120 μL is transferred to the auto sampler (4°C), the auto sampler uses a step function to draw the liquid from the sample and 1 M ammonium acetate solution (4 times 25µl each), just prior to analysis, which is sufficient to serve as a mixing between the two solvents. | Eluting pigments were detected by absorbance (440 nm). Pigments were identified and quantified by comparing their retention times and peak areas with those of pure standards in the library (DHI). Pigment standards were spectrophotometrically calibrated using extinction coefficients according to (Bidigare 2013) | (Barlow et al., 1997) (Bidigare, 1991) (Möller et al., 2024) |
| Peeken_PS74 Peeken_PS76 Peeken_PS78 | HPLC system Waters: auto sampler (717 plus), a pump (600),photodiodenarray detector (2996), a fluorescence detector (2475) and EMPOWER software. reverse-phase HPLC, by the utilization of a VARIAN Microsorb-MV3 C8 column (4.6x100 mm) and HPLC-grade solvent (Merck). The mixture of solvent A was built up of 70% methanol and 30% 1 M ammonium acetate whereas solvent B contained 100% methanol (gradient modified after Barlow et al. 1997). Eluting pigments were detected by absorbance (440 nm) and fluorescence (Ex: 410 nm; Em: >600nm). Retention times served to identify the pigments by comparing them with the retention times of pure algal extracts and pure standards. To assure the identity of each pigment diode array absorbance spectrum (390-750 nm) were compared with the library from the injected standards. Pigment concentration was quantified based on the peak areas of external standards | The filters were homogenized for 20 sec with 50 µl internal standard (canthaxanthin), 1.5 ml acetone and small glass beads in a Precellys® tissue homogenizer. Afterwards a centrifugation was performed in which the supernatant liquid was kept and filtered through a 0.2 µm PTFE filter An aliquot of 100µl was transferred to the auto sampler (4°C) and mixed with 1M ammonium acetate solution (ration: 1:1). Adjacent the liquid was injected onto the HPLC-system | Concentrations of external standards were spectrophotometrically determined using extinction coefficients of Bidigare, (1991) and Jeffrey et al. (1997). A normalization of the pigment concentrations to the internal standard (canthaxanthin) was finally done to counteract possible bias by volume change and experimental losses. | Follows (Barlow et al., 1997), adapted for open ocean and polar waters by (Hoffmann et al., 2006) (Bidigare, 1991) (Jeffrey and Vesk, 1997) |
| Peeken_PS80 | As Peeken_PS74,Peeken_PS76, Peeken_PS78 but Waters: auto sampler (717 plus) replaced by OPTIMAS™ autosampler | As Peeken_PS74,Peeken_PS76, Peeken_PS78 | As Peeken_PS74,Peeken_PS76, Peeken_PS78 | As Peeken_PS74,Peeken_PS76, Peeken_PS78 |
| Peeken_PS85 Peeken_PS92 | As Peeken_PS74,Peeken_PS76, Peeken_PS78, but Waters (600) pump was replaced by 1525 binary pump | As Peeken_PS74,Peeken_PS76, Peeken_PS78 | As Peeken_PS74,Peeken_PS76, Peeken_PS78 | As Peeken_PS74,Peeken_PS76, Peeken_PS78 |
| MOSAiC (PS122) | As Peeken_PS74,Peeken_PS76, Peeken_PS78 | Prior to extraction according to As Peeken_PS74,Peeken_PS76, Peeken_PS78, filters were freeze-dried at -55°C for 48h. | Freeze-drying (FD) significantly and equally improves the extraction efficiency of all pigments. An additional benefit of freeze-drying is that it improves the reproducibility of the extraction procedure, especially when multiple people are involved in collecting samples. | (van Leeuwe et al., 2006) |
| Baysys |  |  |  | Page 105-106 (Matthes, 2021) |

References

Aiken, J., Pradhan, Y., Barlow, R., Lavender, S., Poulton, A., Holligan, P., and Hardman-Mountford, N.: Phytoplankton pigments and functional types in the Atlantic Ocean: A decadal assessment, 1995–2005, Deep Sea Res. Part II Top. Stud. Oceanogr., 56, 899–917, https://doi.org/10.1016/j.dsr2.2008.09.017, 2009.

Álvarez, E., Losa, S. N., Bracher, A., Thoms, S., and Völker, C.: Phytoplankton Light Absorption Impacted by Photoprotective Carotenoids in a Global Ocean Spectrally-Resolved Biogeochemistry Model, J. Adv. Model. Earth Syst., 14, e2022MS003126, https://doi.org/10.1029/2022MS003126, 2022.

Barlow, R. G., Cummings, D. G., and Gibb, S. W.: Improved resolution of mono- and divinyl chlorophylls a and b and zeaxanthin and lutein in phytoplankton extracts using reverse phase C-8 HPLC, Mar. Ecol. Prog. Ser., 161, 303–307, https://doi.org/10.3354/meps161303, 1997.

Bidigare, R. R.: Analysis of Algal Chlorophylls and Carotenoids, in: Marine Particles: Analysis and Characterization, American Geophysical Union (AGU), 119–123, https://doi.org/10.1029/GM063p0119, 1991.

Bracher, A., Taylor, M. H., Taylor, B., Dinter, T., Röttgers, R., and Steinmetz, F.: Using empirical orthogonal functions derived from remote-sensing reflectance for the prediction of phytoplankton pigment concentrations, Ocean Sci., 11, 139–158, https://doi.org/10.5194/os-11-139-2015, 2015.

Hoffmann, L. J., Peeken, I., Lochte, K., Assmy, P., and Veldhuis, M.: Different reactions of Southern Ocean phytoplankton size classes to iron fertilization, Limnol. Oceanogr., 51, 1217–1229, https://doi.org/10.4319/lo.2006.51.3.1217, 2006.

Jeffrey, S. and Vesk, M.: Introduction to marine phytoplankton and their pigment signatures, Monogr. Oceanogr. Methodol., 1997.

Lee, Y., Min, J.-O., Yang, E. J., Cho, K.-H., Jung, J., Park, J., Moon, J. K., and Kang, S.-H.: Influence of sea ice concentration on phytoplankton community structure in the Chukchi and East Siberian Seas, Pacific Arctic Ocean, Deep Sea Res. Part Oceanogr. Res. Pap., 147, 54–64, https://doi.org/10.1016/j.dsr.2019.04.001, 2019.

Lee, Y., Cho, K.-H., Jung, J., Yang, E. J., Moon, J. K., and Kang, S.-H.: Climate-driven changes in phytoplankton community structure in the Northern Chukchi and East Siberian Seas, Pacific Arctic Ocean, Environ. Res. Commun., 7, 091005, https://doi.org/10.1088/2515-7620/ae009c, 2025.

van Leeuwe, M. A., Villerius, L. A., Roggeveld, J., Visser, R. J. W., and Stefels, J.: An optimized method for automated analysis of algal pigments by HPLC, Mar. Chem., 102, 267–275, https://doi.org/10.1016/j.marchem.2006.05.003, 2006.

Matthes, L. C.: Light propagation in ice-covered environments: Seasonal progression and biological implications, 2021.

Möller, K., Thoms, S., Tillmann, U., Krock, B., Koch, F., Peeken, I., and Meunier, C. L.: Effects of bottom-up factors on growth and toxin content of a harmful algae bloom dinoflagellate, Limnol. Oceanogr., 69, 1335–1349, https://doi.org/10.1002/lno.12576, 2024.

Parsons, T., Maita, Y., and Lalli, C. M.: Amanual of chemical and biological methods for seawater analysis, Biol. Oceanogr. Process., 173, 1984.

Stoń-Egiert, J. and Kosakowska, A.: RP-HPLC determination of phytoplankton pigments—comparison of calibration results for two columns, Mar. Biol., 147, 251–260, 2005.

Ston-Egiert, J., Lotocka, M., Ostrowska, M., and Kosakowska, A.: The influence of biotic factors on phytoplankton pigment composition and resources in Baltic ecosystems: new analytical results, Oceanologia, 52, 101–125, 2010.

Taylor, B. B., Torrecilla, E., Bernhardt, A., Taylor, M. H., Peeken, I., Röttgers, R., Piera, J., and Bracher, A.: Bio-optical provinces in the eastern Atlantic Ocean and their biogeographical relevance, Biogeosciences, 8, 3609–3629, https://doi.org/10.5194/bg-8-3609-2011, 2011.

Tilstone, G., Dall’Olmo, G., Hieronymi, M., Ruddick, K., Beck, M., Ligi, M., Costa, M., D’Alimonte, D., Vellucci, V., Vansteenwegen, D., Bracher, A., Wiegmann, S., Kuusk, J., Vabson, V., Ansko, I., Vendt, R., Donlon, C., and Casal, T.: Field Intercomparison of Radiometer Measurements for Ocean Colour Validation, Remote Sens., 12, https://doi.org/10.3390/rs12101587, 2020.

Zapata, M., Rodríguez, F., and Garrido, J. L.: Separation of chlorophylls and carotenoids from marine phytoplankton: a new HPLC method using a reversed phase C8 column and pyridine-containing mobile phases, Mar. Ecol. Prog. Ser., 195, 29–45, https://doi.org/10.3354/meps195029, 2000.

Zindler, C., Bracher, A., Marandino, C. A., Taylor, B., Torrecilla, E., Kock, A., and Bange, H. W.: Sulphur compounds, methane, and phytoplankton: interactions along a north–south transit in the western Pacific Ocean, Biogeosciences, 10, 3297–3311, https://doi.org/10.5194/bg-10-3297-2013, 2013.